GUIDELINES

ON

NON-STRUCTURAL MEASURES IN

URBAN FLOOD MANAGEMENT

Ivan Andjelkovic

1. Origin, status and perspectives of flood management

1.1. Origin of floods

Floods are natural disasters that have been affecting human lives since time immemorial.

Throughout history, nature has shown little respect for man's unwise occupancy of nature's

right-of-way and has insured that the message has been clearly understood by sporadically flooding people's properties and taking their lives.

Flood are associated with some extreme natural events that happen on a geographical area known as a drainage basin, which is also referred to as a river basin, a catchment area or a watershed. Drainage basins can be rural (natural) and urban, the former commonly being much larger than the latter. Hence, flooding can be rural and urban. One of the goals of these Guidelines is to clarify the difference between floods in those two environments.

An extreme natural event only becomes a disaster when it has an impact on human settlements and activities. Therefore, there is both a strong social and natural science component to floods.

The basic cause of rural or river basin flooding is heavy rainfall or rainfall combined with snowmelt, followed by slow development of flood flows, which exceed the capacity of natural waterways. Other causes of rural floods are:

- surcharge in water levels due to natural or man-made obstructions in the flood path (bridges, gated spillways, weirs)

- sudden dam failure

- landslide

- mud flow

- inadequate urbanisation (excessive encroachment in the floodway)

- ice jam

- rapid snowmelt

- deforestation of the catchment basin

Rural floods are river-basin events, whereas urban floods can have both area-wide and local origin, and are accompanied with serious water pollution problems. Urban flood occurrence is not bound by local administrative boundaries because stormwater drainage and protection facilities are part of an environmental system that is larger than an incorporated city territory.

A large portion of the world's population lives along river banks because the streams, aside from providing water for human use, industrial production, and sanitation, have built over geological time alluvium deposits that created the best agricultural lands. Urban settlements organised the protection against flooding by building dikes and river training works, whereas most of the land behind the line of protection has remained unchanged.

Streams carried eroded material from the upper parts of the watershed into the lower segments of the river course. Alteration of the sediment transport pattern in the rivers caused sediment deposition and gradual elevating of the natural riverbed. The land adjacent to the river became higher in elevation than the land farther away due to the continuous deposition of the eroded material. In such a way, natural water stages became elevated as well and flooding of the riverbanks was made possible.

1.2. Different aspects of flooding

Flooding has several aspects, such as climatic, social, economic, institutional, and technical, that are differently addressed for rural and urban conditions.

The climatic of flooding deals with the climatic conditions that may lead to the occurrence of floods. In urban conditions, short and intensive showers proved to be just as critical as long lasting rains, but in rural conditions long lasting rains over an area-wide territory, accompanied with snow melting in the river basin, are recognised as possibly more influential.

The social of flooding deals with the way the floods occur in different settings. In urban conditions, one can negotiate the intensity and frequency of the disruption of public life and

traffic, whereas in regional conditions the common term is disaster, although there were many situations where local urban flooding had disastrous consequences (casualties and property losses) as well. However, floods do not necessarily always need to be associated with disastrous consequences.

The economic of flooding deals with the issues of financing the capital improvement, operation, and maintenance of flood protection schemes. Local stormwater drainage and flood protection is usually financed by local revenues, such as local taxation, service fee, or user charge fee, collected on the basis of land use, whereas the regional protection is mostly carried out through general taxation.

The institutional of flooding deals with the role of governments in the process of decision making. In local conditions all major decisions are made by local governmental institutions and water-related companies, whereas in regional issues federal government and ministries take over the full responsibility. Increasing participation of non-governmental organisations is becoming noticeable as well.

The technical of flooding deals with the concepts and works usually applied in flood protection. In urban conditions, the "dual drainage" concept is most commonly applied, introducing the distinction between the stormwater drainage service and urban flood protection, whereas in area-wide conditions flood control measures are always regarded as a part of the regional or state-wide flood control schemes.

1.3. Flooding in urban environment

Cities concentrate production and population and provide some obvious advantages over rural settlements. Those are, among many, lower specific cost for the provision of potable water supplies, sewers and drains, garbage collection, telecommunication, transportation and most forms of health, educational and emergency services, as well as the possibilities of collecting taxes and charging the public for providing different communal services. On the other hand, the concentration of domestic, commercial and industrial wastes causes major environmental and health problems for the city inhabitants that are spread by water movement. Principal phases of the urban water cycle are given in fig. 1.1.

Fig. 1.1. Movement of water in urban environment

In the past, due to the absence of planning, expansion of cities took place on the best quality farmland in the vicinity of the waterways. The result of that process is the present situation in cities of both the developing and developed countries, where poorer, but larger, segments of population live on land which is ill-suited to housing, because those are the only land sites they could have afforded. People that occupy the high lands in cities right fully take the advantage of their favourable elevated position, but very often the level of their financial participation in flood control is below the level of their contribution, expressed in monetary units, in worsening the problem of urban flooding.

Cities have been permanently developing their water-related infrastructure and discharging their urban waters into the nearest water body. During time, natural, undisturbed, discharging conditions were becoming deteriorated due to the raising of the river water stages.

The problems of discharging urban waters became particularly expressed at the sites where waste water and storm water were being conveyed in common trunks, due to submergence of the outfalls and possible back-flow during high water stages. It is not uncommon that normal flood control measures between the flood protection dikes are performed simultaneously with the emergency, near-to-accident, measures behind the dikes, where mixed waste and storm waters overflow into basements or flow out onto the streets through street grated inlets (fig 1.2). Urban waters proved to need an integral treatment.

Fig. 1.2. Possible different flooding conditions in urban environment

The primary cause of urban flooding is a severe thunderstorm or a rainstorm proceeded by a long-lasting moderate rainfall that saturates the soil. Floods in urban conditions are flashy in nature and occur both on urbanised surfaces (streets, parking lots, yards, parks) and in small urban creeks that deliver water to large water bodies. Other causes of urban floods are:

inadequate land use and channelisation of natural waterways

failure of the city protection dikes

inflow from the river during high stages into urban drainage system

surcharge due to blockage of drains and street inlets

soil erosion generating material that clogs drainage system and inlets

inadequate street cleaning practice that clogs street inlets

Floods disrupt the social systems of the countries and the cities, and cause enormous economic losses. Impacts produced by increased runoff in urban setting are the following:

loss of human life

flooding of housing, commercial and industrial properties

flooding of streets, intersections and transportation systems, causing traffic delays

recurring basement backups from surcharged sanitary sewers

inflow of stormwater into sanitary sewers

municipal waste water treatment plant by-passing

combined sewer overflows

spilling the surcharged sewers content into streets

damage to public and personal property

health hazards

disruption of services such as water supply, sewerage and power supply

delays in public transportation

cleanup demands

adverse effects upon the aesthetics

disturbance of wildlife habitats

economic losses

pollution of local waterways and receiving water bodies

1.4. Hydrologic impacts of urbanisation

Urbanisation has many definitions. One of the simplified ones describes urbanisation as a process of artificial land use alteration during time. Temporal dimension of urbanisation is usually quantified by a design period, defined as a period of time in which reliable forecast  of urban changes can be made. It usually ranges between 15 and 25 years.

Conversion by people of pervious natural surfaces to less- or non-pervious artificial surfaces is responsible for increases in both the stormwater runoff rates and the total runoff volumes, due to decreasing the natural water storage capacity of the soil. Natural storage in a watershed is being made available by the effects of infiltration, vegetation wetting, interception and depression storage. An often neglected fact is that change of natural water storage as a consequence of urbanisation, also causes significant changes to the temporal characteristics of runoff from an urbanised area, such as shortening the runoff travel time and giving to the event a flashing appearance.

Urbanisation is also responsible for an alarming increase of pollutants in natural water bodies. Stormwater runoff may contain organic wastes, nutrients, bacteria, suspended solids, heavy metals, oils, animal waste, tire and vehicle exhaust residues, de-icing chemicals, street litter, and sediments from construction sites. Improvements in sampling and monitoring techniques indicated that there is a rapid increase in concentration of pollutants which occurs throughout the storm event. Effects of urbanisation on urban waters are simplified in a graphical form in fig. 1.3.

In some ancient civilisations, stormwater was stored and preserved. However, during the industrial revolution the stormwater has been considered a nuisance in urban life and water was evacuated from the city areas as fast as possible. Urbanisation changed natural runoff pattern and accelerated transport of water, pollutants and sediment from the urban areas. At the same time, effects of the rural areas on natural waters changed significantly by deforestation and by the introduction of fertilisers in agriculture.

The traditional “efficient conveyance” approach was shifted gradually towards the “water storing” approach, focusing on detention, retention and recharge. However, since stormwater was becoming a significant source of pollution, new concepts of source control, flow attenuation and treatment in natural and artificial biological systems were introduced.

Fig. 1.3: Hydrological impacts of urbanisation

There is no doubt that in near future urban stormwater will become again a precious resource for man's survival.

Stormwater may be discharged in a number of ways. Pipes, channels, and drainage ditches are the most obvious ones. But stormwater discharges can also appear in a form of small rivulets in dirt roads, swales, infiltration soakways, porous pavement, and different

depressions on the ground surface. Even "sheet flow", like that across a flat area, is considered a stormwater discharge if it ends in a storm sewer or any receiving water body.

Urban stormwater control systems have two components: (i) the system "minor", a

stormwater drainage system conceptualized on the basis of a pre-determined level of accepted risk, composed of sewers, open channels, curbs, gutters, inlets and swales, and (ii) the system "major", which operates whenever the capacity of the other system is exceeded and is composed of natural and man-made flow paths that lead excess overland flow towards the receiving water bodies. Many communities have not recognised that system major existed until a severe runoff event occurred. To their dismay they discovered that system “major” included their homes, basements, industrial areas, parks, and commercial buildings constructed in low lands.

The problems caused by excessive precipitation in urban setting are ordinarily grouped in five categories:

- stormwater drainage and flooding control

- soil erosion

- sedimentation of eroded material

- pollution of land and water bodies

- interference with groundwater supply

1.5. Perspectives of flood management

Floods cannot be prevented but planning the emergency measures through flood management can often reduce their disastrous consequences.

Inception of flood management normally begins after a major flood event. People always have some other priorities until a major disaster happens. Throughout  history, progress in water-related disciplines often came as a reaction to severe emergency situations : water supply systems were extended after major droughts, sewerage systems upgraded after outbreaks of major epidemics, pollution control schemes introduced after major spills of pollutants that destroyed life in water courses, and stormwater drainage systems built after major floods.

Flood management is a broad spectrum of water resources activities aimed at reducing potential harmful impacts of floods on people, environment and economy of the region. The main limitation of the current flood management methodologies comes from favouring mostly economic impacts and paying minor attention to the environmental and social impacts of floods.

Current urban flood management practice includes an non-quantified safety factor.

Because statistically homogeneous measured data from the past is generally lacking, one cannot predict frequencies of occurrence of various runoff features such as peak flows, volumes, overflows etc. However, there are places where reliable precipitation and runoff data are readily obtainable. The fact that a particular outflow can result from many combinations of hydrologic conditions is often neglected. Two identical rainfalls do not necessarily create identical runoffs, because of possible different antecedent conditions in the drainage basin.

Therefore, flood management policies should be basically founded on mathematical modelling and sound engineering practice rather than on statistic and probabilistic analyses.

Total flood protection is unrealistic and unwise. The ultimate goal of flood loss prevention is the improvement of the quality of life by reducing the impact of flooding and flood liability on individuals, as well as by reducing private and public losses resulting from the flooding. The objectives of the urban flood management are to provide answer to the question of how to deal effectively with the possibility of flooding in urban environment and how to cope with the associated uncertainties.