Caving Performance Through Integration of Micro-Seismic Activity and Numerical Modeling at DOZ – PT Freeport Indonesia

Rubio, Enrique

Mining Engineering Department, Universidad de Chile, Santiago, Chile

Napitupulu, Daulat

Geotechnical Engineering Department, PT Freeport Indonesia, Tembagapura, Indonesia

ABSTRACT:  This paper describes an undergoing research at PT Freeport Indonesia in which the main goal is to use the micro seismic information recorded as a result of mining to analyse cave propagation and stress performance on the actual production and fixed infrastructure. At the moment, several numerical experiments have been conducted in order to correlate the mining activity with the micro seismic events using data collected during year 2005 and 2006. As a result of the preliminary analysis a micro and a macro cracking envelop is proposed based on the computation of stress behaviour at the location of the events. Stresses have been computed using standard elastic continuous boundary element models. The correlation between average source radius together with the stress performance have provided a method to propose a macro cracking criteria. Several techniques have been tested in order to nucleate the micro seismic activity around different geological features. This last attempt was aimed to look at potential over stresses induced over the undercut and extraction level drifts. A method was devised in order to integrate the micro seismicity into a 3 dimensional ride distribution model. This model has shown to be very effective to quantify the over stress induced due to volumetric micro seismicity. The volumetric micro seismic model showed to induce up to 10 MPa of over stress over a period of two months. The future work will be concentrated on calibrate the integrated model with actual damage observations made at the current mining infrastructure.

# InTRODUCTION

Cave mining induces micro seismicity in the rock mass as a result of the fracturing process that takes place above and below the production and haulage level as reported by Glazer and Hepworth (2004) and Dunlop and Gaete (1995).  Micro seismicity induced by mining at high stress in brittle rock masses has been researched for years in order to find a pattern between micro seismic activity and violent failure in a form of rock burst as shown by Simser and Falmagne (2004) and also Beck and Brady (2002) have proposed a parametric formulation to evaluate the controlling parameters for seismic events in hard rock mining.

In block and panel caving mining there is always a need to better understand the caving propagation as a well as the stress distribution on the undercutting and production drifts to assess production performances and long term stability of the production facilities.

This paper addresses the fact of whether the micro seismic activity may be used to better understand the caving process. In particular, it was sought a relationship to assess cave propagation. This was performed by combining the micro seismic activity of

DOZ, PT Freeport Indonesia mine and numerical modeling. The second question that the paper intends to address is whether the micro seismic activity is reporting some insights regarding rock mass behaviour that are not possible to represent in a single elastic continuos model.

A fair amount of work has been conducted in the latest years on micro seismicity induced by caving mining. The technique has been refined by adding different performances to the current practice of micro seismic monitoring. Different performance indicators such as magnitude, energy index, stress drop and others have been derived over time. Nevertheless very little mining interpretation has been given to these indicators. Even more, not a clear analogy between caving performance and seismic indicators has been established. The research presented in this paper intends to over come a few of these aspects by integrating seismic activity into numerical modeling. This work aims to develop a methodology to use micro seismic information to calibrate numerical models to be used in block cave mining.

A cave propagation factor as well as a 3d ride distribution model have been devised as a result of integrating micro seismic activity in a block cave numerical model. The indicators have been tested at DOZ mine of PT Freeport Indonesia, several comparisons and analysis are shown that these indicators can well be used to predict caveability and potential damage on current production and fixed infrastructure.

# Micro-seismicity at DOZ

DOZ is the underground mine of PT Freeport Indonesia located on the West Papua area. The mine currently produces about 50,000 tpd. The mining method consist of panel caving with advanced and post undercutting . Since July 2004 the mine has been recording micro seismic activity in order to better understand the cave propagation process as well as to find indicators that may guide the occurrence of large deformations on the mining infrastructure.

The micro seismic network of DOZ is composed out of 18 seismometers and 36 triaxial geophones (30 Hz, 14 Hz and 4.5 Hz). The magnitude sensitivity of DOZ micro seismic network was set up at -3.0 and the location accuracy of the network is 10 m – 20 m for elev. 3100 to elev. 3800.

The range magnitude of seismic events recorded by the system were -3 to 3.3 during 2004 – 2007.

The micro seismic database used in the research was selected based on the reliability of the network that was achieved by the end of 2004. Also the micro seismic data was filtered to consider those events that are inside the network envelop to avoid the mixing of data with different location accuracy. Figure 1 illustrates the filtering process.

The filtering process reduced from about 85.000 down to 45.000 events located from January 2005 to March 2006. The data that has been included in the study is from January 2005 until March 2007. A summary of the main seismological parameters are summarized in Table 1.

Table 1. Micro seismic events source parameters statistics

Figure 1. Filtering seismic events using the 3D representation of the seismic network

Magnitudes are estimated using a relationship that has been calibrated to obtain a linear Gutenberg – Richter relationship.

Magnitud=0.274log(E)+0.375logM−4.4

From Table 1 is seeing that no measure events have been developed at DOZ, mainly because of the field stress and the rock mass behaviour. Just 1130 events have been grater than 0 . Mainly the micro seismic activity is associated to cave propagation and abutment stress at the cave boundary.

# Cave Propagation Through Micro-Seismicity

The main geotechnical parameters affecting the planning of the block cave are presented by Brown (2003) as follows:

- Caveability

- Cave initiation

- Cave propagation

- Fragmentation

- Stress performance surrounding the cave boundary

Cave propagation is a very important geotechnical assessment for block cave mining since it constraints draw rates and mining sequence. Cave propagation is called to the fracturing process that occurs at the cave back. Several attempts have been made in order to predict cave propagation. However Szwedzicki et. al. (2004) reported an empirical chart to predict cavability as a result of studying TDRs breaking at DOZ. The approach taken in this research was to use the micro seismic information to understand the stress behaviour at events locations.

The first numerical experiment consisted in construct a mine wide boundary numerical model that could represent the mining geometry and the pre-mining stress state. The numerical model was constructed using the commercial package MAP3Di, that uses boundary elements to resolve equilibrium. This software provides an efficient way to reproduce the complex caving mining geometry. The elements that were included in the model are listed as follows:

- Previous mining represented by IOZ cave

- Actual mining represented by DOZ cave back

- Actual undercut and production level drifts

- Undercut progression boundary, UCL

Figure 2 shows the component of the numerical models.

The measure rock unit that is present at the mine during the time under study is Skarn that has the elastic parameters of a young Modulus of 52 GPa and a Poisson ratio of 0.28.

The pre mining stress state was estimated based on the available pre mining stress state that is represented by the following stress tensor.

Table 2. Pre mining stress parameters

After constructing the numerical model the micro seismic activity associated with the mining geometry was loaded in the model in order to compute the stress components at all the events locations. Figure 3 shows the micro seismic events recorded during January and February 2005 embedded into the numerical model.

The same numerical experiments have been reported by Beck (1998) and Martin (1994) both authors reproduced the mining excavation geometry and compute the stresses at each event location. Beck at Brunswick mine, after removing those events that were associated with existing geological structure, found that these stresses could be fit by a linear regression given by s1 - s3 » 30 MPa. At AECL’s underground research laboratory, it was found that these stresses could be fit by a line given by s1 - s3 » 60 MPa. Thus, both authors have found linear relationship between the compressive and the confinement component of the stress behaviour at the source. The results of performing the same experiment at DOZ using the January – February 2005 micro seismic activity is shown in Figure 4. The regression of the stress state at the micro seismic events is s1 - s3 » 20 MPa.

Figure 3. January 2005 DOZ Mine Geometry and associated Micro seismic activity

Figure 4. Seismic Envelop May-Jun 2005

After computing the seismic envelop for every event it was computed the difference between the computed stress state (s1) and the regressed stress. Then this difference was displayed in the numerical model in order to identify the locations of larger deviations respect to the regressed stress state.

Figure 5 shows that the most deviated values are associated with two geological faults and the stress concentration at the cave boundary which is not reflecting the caving propagation process. By filtering these two events the standard deviation of the regression showed above could improve from 14% to 3%.

Figure 5. Square difference of stress state between the model and regression

The above numerical experience was repeated every two months and the results are shown consistency in presenting similar regression.

Martin (1997) defines three strength parameters determined from laboratory triaxial compression tests: crack initiation (ci); crack damage (cd), and peak strength (f). In which the following relationship is proposed.

2.5σi=σf1.25σd=σfThe seismic envelop σ1−σ3=20 represents the state of the stress for the crack initiation process (ci). This parameter is usefull to define the befinning of caving. Nevertheless it is necessary to identify the crack damage (cd) strength to better assess the cave propagation. In order to compute this point it was decided to study the micro seismic source parameters further, in particular, the seismic radius and source displacement. McGarr (1984) proposed a formulation to compute these two parameters at the source. Seismic average displacement at the source u=M/Gπr2. Seismic radius at the source r=4M/3Δσ1/3. These relationships were applied to all the events recorded in the period under study. For a given numerical model representing the mining geometry at the time (September 2005) the events recorded in that period and the subsequent month were imported into the model. A vertical 2D voxel model (20x20x20 cell size) was set up in order to interpolate the average micro seismic source radius. Figure 6 shows the vertical section voxel model to interpret the average source radius.

Based on Figure 6 it is possible to establish that a macro crack is forming if the average source radius in the voxel is greater than the voxel size. Based on this criteria the area of potential macro cracking is outlined and the stress performance is computed within this area.

Figure 6. Average source radius in a 20x20x20 voxel for September 2005

Figure 7 shows the over stress re spect to the seismic envelop or the micro cracking criteria. It is found that at an outline of 1.2 there is macro cracking formation. Based on this result it is possible to propose the crack damage criteria to be as follows:

σcd=1.2σ3+24

Figure 7. Over stress performance as a function of the seismic envelop

Finally the cave propagation factor used to predict caveability is shown as follows:

CPF=σ11.2σ3+24If CPF≥1 there is caving otherwise the rock may be cracking but it is not caving trough surface. This criteria was used to predict the cave propagation as June 2006 as shown in Figure 8. This estimated cave propagation as been compared against caving observed at pre existing tunnels in ordere to confirm the estimate.

Figure 8. CPF used at DOZ to predict June 2006 cave back location

# Micro Seismic activity as a guide for Induced stresses

One of the main questions that arose at DOZ was whether the micro seismic activity was reflecting different stress paths that are not considered by the simple elastic linear models. In other words, by integrating the micro seismic activity will production and fixed infrastructure eventually be at danger as a result of stress concentration that is not reflected by actual mine geometry. Also the question was whether it is possible to integrate the micro seismic activity into numerical models to have a common indicator of damage. This was a result of looking at different back analysis and find that neither the energy index, nor the magnitude was a good estimator of the damage observed underground. The first numerical experiments consisted on analyzing the stress effect of the micro seismic activity associated to an existent fault on surrounding infrastructure. The second approach consisted on integrating the whole micro seismic activity in a 3D ride distribution model to analyze differences of stress path at the cave boundary.

## Induced stress as a result of the micro seismic activity associated to a fault

The integration of micro seismicity with numerical modeling starts by computing the average displacement and radius of every seismic event at the source. These two parameters were computed and plotted in to the calibration chart shown in Figure 9. This chart confirms the relationships used to compute these two source parameters.

Figure 9. Source radius[m] versus logarithm of seismic moment [Nm] for different stress drop[MPa]

To interpret the effect of the micro seismicity associated to a fault the first step consisted in computing the energy index (Simser and Falamagne, 2004) and make a contour plot to identify the area surrounding the fault that should be included in the model. Figure 10 shows a plan view of the energy index associated with the micro seismic events

Figure 10. Energy index contour surrounding the West fault.

As discussed by Walsh and Waterson (1987) and Hoffman et. al. (2000) one could consider two kind of ride distribution models elliptical crack or asperity with soft edges. It is not the purpose of the paper to discuss which ride distribution model should be used, in fact, to model the ride distribution induced by micro seismic events associated to the fault there should be used the asperity model and for the cave associated events the elliptical crack model. For simplicity at this stage of the research it was used just the elliptical crack model which is presented by:

u(r)=1.5D1−r/a20.5

Where D is the average displacement at the source, a is the source radius and r is the average rip.

Based on the energy index nucleation method a 2D voxel model was constructed to plot along the fault. The extent of the voxel was set up to be 50 m each side of the fault and 10x10 m voxel sizes.

The ride distribution model is imported into MAP3Di software to solve equilibrium incorporating it as boundary condition. It was expected that a great amount of stress would be induced on the fault abutments. Nevertheless, it was found that very little shear stress was induced on the abutments as a result of integrating the micro seismicity. It is estimated that the measure differences in shear stress would be about 2.5 MPa. Several areas on the undercut level were selected in order to find a relationship between the induced displacement and the shear stress difference as a result of including the micro seismic events. Figure 11, shows a chart showing difference of shear stress as a result of integrating induced displacement inferred from the micro seismic events.

In summary, the yielding activity of the fault produces almost no effect on the stress field surrounding the production and undercutting drift infrastructure. Mainly because the small size displacements observed at this geological feature and represented by the micro seismic events recorded at the mine.

Figure 11. Induced shear stress as a result of integrating the micro seismic activity asociated to the West Fault.

## 3D ride distribution model to integrate micro seismicity induced by caving in a numerical model

The challenge on this part of the research was find a method to integrate the 3 dimensional characteristics of the micro seismicity induced by caving. Currently, there is a fair amount of research done regarding integrating micro seismic activity related to fault slip and general energy release in the process of fault displacement. However the micro seismicity recorded as a result of mining is not just located at the geological features. Events are located at places where caving is occurring, as it was shown before in this paper.

The approach taken in this part of the research was to split the micro seismic activity into different plan layers. For every layer a 2D ride distribution model would be performed using the elliptical crack interpretation. Then all layers would be integrated into a single file, Figure 12 shows the micro seismic 3D ride distribution model constructed using the micro seismic activity recorded in September-October 2005.

Figure 12. 3D ride distribution model of September-October 2005.

Finally the 3D ride model is imported in MAP3Di in which the displacements inferred from the micro seismic activity are used as a boundary condition for solving equilibrium. The difference on shear stress concentration of integrating the 3D ride model versus simulating the excavation with a simple elastic model is shown on Figure 13.

Figure 13. Effect of 3D ride model on shear stress distribution

Based on the results shown on Figure 13 it was decided to compute the effect of the micro seismic activity as a function to the distance to the cave boundary in order to assess whether there could be a stability hazard or not. Table 3 shows the change on shear stress as result of integrating the micro seismic activity.

Table 3. Difference on Shear stress distribution as a result of integrating micro seismic activity.

The results shown above are relevant since they are indicating that the integration of micro seismicity is giving important insights regarding rock mass behaviour that it is not trivial to integrate in an elastic linear model. At the cave boundary the difference on shear stress in model with versus without micro seismicity vary in the range of 7.9 and 10 MPa. This result is enough to find some areas at medium to high damage depending on the location of the area across the mining layout.

# Outstanding Issues

The micro and macro cracking envelops are to be calibrated using the damage observations on the mined out levels that are located at higher elevations than DOZ production level.

The seismic envelop has to be divided into the shear fractures and micro cracks in order to better assess the regression of macro crack formation.

Voxel geometry as well as layer spacing to create the 3D ride distribution model need to be further study to analyse the impact of these parameters into the overall induced stress.

Further research is taking place at the moment to calibrate the micro seismic integrated models. The calibration is taken place by compiling the damage observation with the stress performance computed by the numerical model.

# Conclusions

Cave propagation can be estimated based on a macro cracking criteria that has been devised integrating elastic linear models with the recorded historical micro seismic activity. Crack initiation also can be estimated with the direct correlation of compressive and confinement stress estimated at the source using numerical models.

At DOZ at the moment there is still uncertainty regarding how to use the micro seismic information to assess potential production and fixed infrastructure damage. It is difficult to observe direct correlations between source parameters such as moment, energy, energy index or magnitude and damage. The integration of micro seismic information into numerical modeling provides a way in which field observations can be correlated with stress performance. In particular, using simple relationships such as over stress for rock mass and excess shear stress for faults.

# Acknowlegdements

Authors would like to acknowledge PT Freeport Indonesia for supporting this project and facilitate the edition of this paper. Special acknowledgments should be given to Mr. Srikant Annavarapu General Superintendent of Geotechnical Underground engineering for supporting this study. Also acknowledgments should be given to Universidad de Chile for providing the academic support to conduct the research presented in this paper.

# references

Beck D.A., Brady B.H.G, 2002. Evaluation and application of controlling parameters for seismic events in hard-rock mines. International Journal of Rock Mechanics & Mining Sciences 39 (2002) 633–642

Beck DA, Brady BH, Grant DR, 1997. Induced stress and seismicity in the 3000 Orebody, Mount Isa. Geotech Geol Eng 1997;15: 222–33.

Dunlop R and Gaete S, 1995. Seismicity at El Teniente mine: a mining process approach. 4th International Symposium on Mine Planning and Equipment Selection. October 31 –November 03, Calgary, Canada.

Glazer, S. and Hepworth, N., 2004. Seismic Monitoring of Block Cave Crown Pillar – Palabora Mining Company, RSA. In Proceedings, MassMin 2004, pp 565-569, Santiago, (Ed: A Karzulovic and M Alfaro).

Hoffman, G, R. Sewjee and G. van Aswegan, 2000. First steps in the integration of numerical modelling and seismic monitoring, 5th International Symposium on Rockburst and Seismicity in Mines, SAIMM, Johannesburg, 2000, pp. 397-404.)

Martin, 1994 The progressive fracture of Lac DuBonnet Granite , Int. J. Rock Mech. Min. Sci. 31 643-59

McGarr, 1984. Some Applications of Seismic Source Mechanism Studies to Assessing Underground Hazard, Proc. 1st International Symposium on Rockbursts and Seismicity in Mines, SAIMM, Johannesburg, 1984, pp. 199-208, and

Simser B, Falmagne V, 2004. Seismic source parameters used to monitor rockmass response at Brunswick mine. CIM Bulletin ■ Vol. 97, N° 1080

Szwedzicki T, Widijanto E, Sinaga F, 2004. Propagation of a caving zone, A case Study from PT Freeport, Indonesia. In Proceedings, MassMin 2004, pp 508-512, Santiago, (Ed: A Karzulovic and M Alfaro).

Walsh, J.J. and J. Waterson, 1987. Distribution of cumulative displacements and seismic slip on a single normal fault surface, J. Struct. Geol., V. 9, pp. 1039-1046