ON THE APPLICABILITY OF THE MULTINOMIAL PROBIT MODEL

Marcela A. Munizaga

Universidad de Chile

Juan de Dios Ortúzar

Pontificia Universidad Católica de Chile

1.	INTRODUCTION

It has been known for many years that the multinomial Logit (MNL) model is not appropriate when the assumption of independent and identically distributed error terms is not reasonable. Alternative model functions, such as the hierarchical logit (HL) model, have been proposed to deal with the problem of correlation among certain options but only the multinomial Probit (MNP) model is free from all deficiencies. It is interesting to mention that although the MNP estimation is now perfectly plausible the model has only been shyly incorporated into practice.

In this paper we will attempt to clarify some aspects, that maybe obscure, related to the use of MNP models in order to stimulate its use or to provide arguments to avoid its use depending on the case. To do this we have identified fairly simple structures of the covariance matrix of the error terms in some common practical cases, and study how different models (including the MNP) behave in those conditions using Monte Carlo simulation. Our results allow us to recommend the use of Probit models in the presence of correlation when large databases are available. Probit or Heteroscedastic Extreme Value (HEVL) models may be used when heteroscedasticity between alternatives is expected. Finally, in the presence of pure heteroscedasticity, the MNL model seems to work fine but with a Single Element Nested Logit (SENL) or a MNP model, the ratio between standard deviations can also be estimated. This may be very important if mixed revealed (RP) and stated preference (SP) models are required.

The rest of the paper is organised as follows. In section 2 we describe the models and their estimation. In section 3 we identify the structure of the covariance matrix in some common practical cases. The empirical evidence is shown in section 4, and the main conclusions of the paper are summarised in section 5.

2.	MODELS

2.1	Derivation

Depending on the assumptions about the error terms several alternative random utility models of discrete choice can be derived. For example, if it is assumed that the error terms are due to the addition of multiple independent factors, then the Normal distribution can be justified (Ben-Akiva and Lerman, 1985); this assumption leads to the MNP where the probability of choosing alternative i is given by (Daganzo, 1979):

P(i|Cn)=Pr[ejn−ein≤Vin−VjnjÎC rSub { size 8{n} }  \] }  {} # 
 {} # 
= Int rSub {−¥}  rSup {V rSub { size 6{ ital ∈} } -V rSub { size 6{1n} } }  { Int rSub {−¥}  rSup {V rSub { size 6{ ital ∈} } -V rSub { size 6{2n} } }  {K Int rSub {−¥}  rSup {V rSub { size 6{ ital ∈} } -V rSub { size 6{ ital jn} } }  {N \( 0,S rSub { size 8{e-e rSub { size 6{i} } } }  \) }  size 12{de}} }  {} 
} } {			(1)

where Vin is the observed component and in the error term of the utility function of option i for individual n; Cn is the choice set for individual n and N(0,  denotes the multivariate Normal distribution with mean zero and covariance matrix . The above integral cannot be solved, and either simulation or an approximation must be used to evaluate it.

Under certain conditions, a much more simple model may be applied. The MNL model can be derived if it is assumed that the error terms are independent and identically distributed (iid) Gumbel. The Gumbel distribution (Gumbel, 1957) can be justified from two different angles: firstly, it approximates the Normal distribution in the case of independent and homoscedastic error terms; more importantly, it is the asymptotic distribution of the probability that a maximum between several iid variables is larger than a certain value.

This model, which can be considered as equivalent to an iid Probit has the following closed form (McFadden, 1973):

P(i|Cn)=exp(mVin)jÎCnexp(mVjn)	(2)

where  is a scale parameter related to the variance of the error term (m=p6s). This parameter is unidentifiable and can be fixed to any value without affecting the model. It is assumed to be equal to unity in most practical applications. Such a scaling process is also needed in the MNP model, where usually the covariance matrix is scaled (divided) by its first element.

The MNL model has been widely used in spite of being criticised because it is not always reasonable to assume iid error terms. In particular, the famous red bus - blue bus paradox (Mayberry, 1973) illustrates how inappropriate the MNL model can be in the case of (perfect) correlation between alternatives. In the following section we identify some other cases where the iid property does not hold.

To solve some of these problems and avoid using the MNP, some ad-hoc models have been proposed. The HL model does not assume iid error terms, but imposes some restrictions on values of its structural parameters in order to achieve plausible cross-elasticities between alternatives (see Williams, 1977; Daly, 1987). It can be used to model discrete choice when there is positive correlation between the error terms of some mutually exclusive groups of the observations. This leads to a block-diagonal covariance matrix.

The SENL which posits a particular hierarchical structure was proposed by Bradley and Daly (1991) to acknowledge the heteroscedasticity between observations problem, within the theoretical framework developed by Ben-Akiva and Morikawa (1990).

A more recent advance in this line is the HEVL model (Bhat, 1995; Bhat, 1997), where the error terms are assumed to be independent but not identically Extreme Value Type I distributed. As in the Probit case, the model formulation implies an integral that cannot be solved (see equation 3); but in this case it can be evaluated using Gauss-Laguerre Numerical integration (Press et al, 1992). The model was developed to account for the heteroscedasticity between alternatives problem and has additional scale parameters related to the variance; one of them has to be fixed to make the model identifiable.

P(iCn)=w=-¥¥jÎCn,j¹1LVi−Vj+qiwqjλwdw	(3)

2.2	Estimation

The MNL model can easily be estimated by maximising its log-likelihood function as convergence is guaranteed. HL and SENL, however, are not as easy to estimate because convergence is not guaranteed in this case, but there is efficient commercial software available to do it, such as ALOGIT (Daly, 1992) or LIMDEP (Greene, 1995).

The estimation of MNP models used to be a difficult task, but recent advances in simulation techniques and computer efficiency allow to estimate Probit models by simulation with a reasonable effort. The state of the art is to apply the Simulated Maximum Likelihood (SML) approach (Lerman and Manski, 1981) using the Geweke-Hajivassiliou-Keane (GHK) simulator (Börsch-Supan and Hajivassiliou, 1993). This GHK simulator reduces recursively the dimension of the integral, leading to an equivalent problem where repetitions of unidimensional truncate Normal variables are required. The simulated probabilities are unbiased, continual and differentiable. The values lie strictly within the (0,1) interval, and the computing time increases only linearly with the number of alternatives. The equivalent simulation problem is constructed by applying the Choleski transformation to the covariance matrix, after deviating with respect to the chosen alternative on each observation. We implemented this estimation method in GAUSS (Aptech Systems, 1994) using code for the GHK simulator downloaded from Vassilis Hajivassiliou Web Site.

As mentioned before, the HEVL probabilities cannot be estimated directly, but Gauss-Laguerre numerical integration can be applied. To do this, the integral has to be re-arranged to the form:

u=0¥G(u)e−udu	(5)

which can be done by means of a simple change of variables. The evaluation points are determined by the roots of the order-k Laguerre polynomial. We implemented this model in GAUSS with 64 evaluation points.

3.	STRUCTURE OF THE COVARIANCE MATRIX

In order to justify the use of a model different than the MNL, we require the presence of some sources of correlation or heteroscedasticity. The error term can be seen as the addition of: the modeller’s incapacity to observe all the variables influencing the decision, measurement errors, differences between individuals, incorrect perception of the attributes, and the randomness inherent to human nature. From this point of view, even the way of modelling and collecting data induces certain structures in the error terms of the utility function. However, due to the presence of mode specific constants the mean of the error term has no influence and the analysis will be centred on the presence of either heteroscedasticity or correlation.

We have mentioned already a clear source of correlation, which is the existence of alternatives that are perceived as more similar than others by the decision maker. A limiting case of this source of correlation is the red bus - blue bus paradox, where both alternatives are perceived as identical. Another extreme case is that of the joint mode-destination choice model, where part of the utility function is common to several options. This last case, described in Williams and Ortúzar (1982), illustrates clearly the relation between alternative similarity and correlation. The presence of a common component in the utility function of two alternatives implies the existence of off diagonal elements in the covariance matrix.

Multiple responses on (SP) experiments has been identified as a source of correlation between alternatives (Cirillo et al, 1996; Ouwersloot and Rietveld, 1996; Ortúzar et al, 1997), though there is not yet consensus on a proper way to model this effect. We propose to look at this problem as one of taste variation, where it can be assumed that each individual has specific taste parameters which apply to each observation corresponding to the same individual. A generalisation of the ordinal Probit model has also been used to treat this problem by Yen et al (1994).

Heteroscedasticity between alternatives can be presumed in several cases. Firstly it can be argued that as individuals will have more precise information about the attributes of the alternatives that they use more often, this implies larger variance of the error term for the rest. This effect can also be a function of the way information is collected about the explanatory variables; for example, in an urban mode choice model the attributes of the chosen mode can be obtained from the survey, while those of the rest have to be obtained by identifying the trip on a network model using average speed values. Other sources of this type of heteroscedasticity are the inclusion of non existent alternatives in SP experiments where more dispersion can be expected, and in cases where some of the options are inherently more variable (for example, surface transport compared with underground or train systems with exclusive ways).

Finally, another effect that could arise is the presence of heteroscedasticity between observations. A clear example is the estimation of discrete choice models with mixed RP and SP data. This case was identified by Ben-Akiva and Morikawa (1990) who made the assumption that the only difference between both databases is the variance of the error terms. Another case of heteroscedasticity between observations that might have some importance in the future, is the difference in the level of information that different users have; for example, those users who have access to some kind of electronic on line information will have more accurate perceptions of the attributes than those who do not and who make their decisions based on historical information or in their own perceptions.

As mentioned in the introduction, three of the identified non iid cases have at least one theoretically correct model. In the next section, we will explore how these models behave under simulated iid errors, correlation and heteroscedasticity conditions1.

4.	EMPIRICAL EVIDENCE

In order to study the behaviour of the aforementioned models and especially that of the MNP, some simulation experiments were made using a methodological approach similar to that proposed by Williams and Ortúzar (1982). Several databases where generated following rigorously  the assumptions established by the various discrete choice models. The explanatory variables where randomly generated assuming a Normal distribution for travel time and cost, and a Binomial distribution for the High Income dummy variable. The values for the mean and variance of these distributions were obtained from a real data set. The utility function was also constructed borrowing taste parameters from real data, and adding Normal or Gumbel distributed error terms. In this way, observations can be generated where an hypothetical individual chooses, within a four options choice set, the alternative that maximises a utility function formed by the sum of a deterministic function of the attributes and taste parameters and an error term. Comparisons are made in terms of parameter recovery capacities and quality of response to policy changes.

Results of the estimation of MNL and MNP models in the case of Gumbel distributed errors are shown in Table 1. The parameters of both MNL and MNP models are directly comparable between them and with the reference values, because the variance has been conveniently chosen to make the scale parameter  equal to unity. The reference values (Ref.) are the taste parameters used to generate the data. As can be seen, the results are not significantly different between models. The MNP with ten repetitions of the simulation has a slightly worst log-likelihood than the MNL, while the MNP with 100 repetitions has slightly better ones. It can be noted that when estimating with a larger number of observations the confidence interval of the estimators is reduced, but in all cases the reference values are within them.

The computing time reported corresponds to processing time in a Pentium personal computer, with a non optimised self made code programmed in GAUSS (Aptech Systems, 1994). It can be seen that though processing times are much larger for the MNP than for the MNL, they cannot be considered "prohibitive" at all. In fact, there are reported results of other implementations of GHK-MNP, with processing times that are one order of magnitude smaller (Bolduc, 1994); this probably corresponds to an optimised implementation in a powerful computer.

Table 1 : Estimation of Logit and Probit models

(iid Gumbel distributed errors, s=p6=1,28254983)

|  | Ref. | MNL  obs | MNP 10 rep 1000 obs | MNP 100 rep 1000 obs | MNL  obs | MNP 10 rep 2000 obs | MNP 100 rep 2000 obs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mode 1 | -0.200 | -0.3813 (-1.6) | -0.2766 (-1.3) | -0.2925 (-1.4) | -0.3792 (-2.3) | -0.2605 (-1.8) | -0.2896 (-2.0) |
| Mode 2 | 0.600 | 0.6100 (6.2) | 0.5917 (6.4) | 0.5830 (6.2) | 0.6142 (8.9) | 0.5959 (9.1) | 0.5865 (8.9) |
| Mode 3 | 0.400 | 0.4550 (3.5) | 0.4229 (3.6) | 0.4285 (3.6) | 0.4188 (4.5) | 0.3983 (4.7) | 0.3943 (4.8) |
| Time | -0.010 | -0.0152 (-3.2) | -0.0143 (-3.2) | -0.0144 (-3.2) | -0.0141 (-4.2) | -0.0132 (-4.2) | -0.0132 (-4.2) |
| Cost | -0.004 | -0.0045 (-3.0) | -0.0041 (-3.0) | -0.0043 (-3.2) | -0.0043 (-4.1) | -0.0040 (-4.2) | -0.0041 (-4.3) |
| High Income | 1.500 | 1.5845 (9.1) | 1.4185 (9.2) | 1.4452 (9.3) | 1.6376 (13.6) | 1.4740 (13.6) | 1.4910 (13.7) |
| Average Log-Likelihood |  | -1.30488 | -1.31416 | -1.30375 | -1.30331 | -1.31128 | -1.30298 |
| Iterations |  | 7 | 4 | 6 | 6 | 5 | 5 |
| Time (min) |  | 0.3 | 9.2 | 41.8 | 0.4 | 22.6 | 74.4 |

The same situation arises when the error terms are iid Normal distributed (Table 2). There is no significant difference between Logit and iid Probit models estimated with the same database, and neither are there differences between the same models estimated with databases where the error terms are iid Gumbel or iid Normal distributed. This empirical evidence contributes to support some frequently used assumptions.

When correlation appears into the scene the situation gets more complicated. In Table 3 we summarise the results of the estimation of several models with 8000 observations. This number was used because the parameters related to the presence of correlation (structural parameter of the HL model and correlation coefficient of the MNP) where not well recovered with smaller databases. In the case reported here the analysis of response to policy changes indicates that the best model is the MNP with known correlation (fixed  MNP). The second best is  the MNP with unknown correlation ( is estimated from the data). The response behaviour of the HL model is slightly better than that of the MNL; this maybe due to the poor estimation of the specific constant of Mode 1 in both models. The results of the HL and MNP models seem to improve both in parameter recovery and in response analysis when the sample size is augmented; therefore a useful recommendation would be to use large databases when correlation is expected.

In the presence of heteroscedasticity between alternatives the theoretically correct models are the MNP and the HEVL. However, we have also included the estimation of HL and SENL because there has been some suggestions that they could be useful to model under heteroscedastic conditions (see for example Börsch-Supan, 1990).

Table 2 : Estimation of Logit and Probit models

(iid Normal distributed errors s=p6=1.28254983)

|  | Ref. | MNL  obs | MNP 10 rep 1000 obs | MNP 100 rep 1000 obs | MNL  obs | MNP 10 rep 2000 obs | MNP 100 rep 2000 obs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mode 1 | -0.200 | -0.2853 (-1.2) | -0.2261 (-1.1) | -0.2235 (-1.1) | -0.1870 (-1.2) | -0.1228 (-0.9) | -0.1357 (-0.9) |
| Mode 2 | 0.600 | 0.5176 (5.3) | 0.4918 (5.3) | 0.4951 (5.2) | 0.5841 (8.5) | 0.5617 (8.5) | 0.5654 (8.5) |
| Mode 3 | 0.400 | 0.3246 (2.5) | 0.2776 (2.3) | 0.3075 (2.7) | 0.3608 (3.9) | 0.3269 (3.9) | 0.3434 (4.1) |
| Time | -0.010 | -0.0094 (-2.0) | -0.0092 (-2.0) | -0.0092 (-2.0) | -0.0107 (-3.2) | -0.0104 (-3.3) | -0.0106 (-3.4) |
| Cost | -0.004 | -0.0045 (-3.0) | -0.0040 (-3.0) | -0.0042 (-3.3) | -0.0051 (-4.9) | -0.0046 (-4.8) | -0.0047 (-5.0) |
| High Income | 1.500 | 1.7235 (10.3) | 1.5611 (10.3) | 1.5879 (10.4) | 1.7189 (14.6) | 1.5675 (14.6) | 1.5877 (14.8) |
| Average Log-Likelihood |  | -1.30320 | -1.31273 | -1.30280 | -1.29769 | -1.30594 | -1.29735 |
| Iterations |  | 6 | 6 | 5 | 6 | 5 | 4 |
| Time (min) |  | 0.2 | 13.6 | 37.1 | 0.4 | 22.2 | 59.7 |

Table 3 : Models with correlated errors. 8000 observations

|  | Ref. | MNL | HL | MNP fixed  | MNP free  |
| --- | --- | --- | --- | --- | --- |
| Mode 1 | -0.200 | 0.0387 (0.7) | 0.0782 (1.1) | -0.1467 (-3.2) | -0.2288 (-2.8) |
| Mode 2 | 0.600 | 0.9091 (27.4) | 0.9403 (22.5) | 0.6199 (22.4) | 0.5550 (9.4) |
| Mode 3 | 0.400 | 0.4914 (12.4) | 0.4997 (12.3) | 0.3667 (13.7) | 0.3426 (10.7) |
| Time | -0.010 | -0.0114 (-13.9) | -0.0118 (-13.4) | -0.0092 (-14.0) | -0.0087 (-11.4) |
| Cost | -0.004 | -0.0046 (-16.2) | -0.0047 (-15.7) | -0.0037 (-16.4) | -0.0035 (-13.2) |
| High Income | 1.500 | 1.5897 (27.1) | 1.5907 (27.1) | 1.3833 (28.9) | 1.3882 (26.8) |
|  | 0.710 |  | 0.8380 (7.2) |  |  |
| 2 | 0.720 |  |  | 0.72 | 0.8915 (7.1) |
| Average log likelihood |  | -1.25746 | -1.25735 | -1.26028 | -1.26020 |

The results shown in Table 4 correspond to a case where the first two alternatives had error terms iid Normal with 1=1.2 and the rest, 2=0.6.

Table 4 : Models with heteroscedasticity between alternatives

(2000 observations)

|  | Ref. | MNL | SENL | HL1 | HL2 | HL3 | MNP | HEVL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mode 1 | -0.200 | 0.3800 (2.4) | 0.2953 (0.6) | 0.4853 (1.6) | 0.7627 (2.2) | 0.2565 (1.4) | -0.1437 (-0.7) | -0.1389 (-0.5) |
| Mode 2 | 0.600 | 1.1292 (15.2) | 1.0469 (2.3) | 1.1990 (4.8) | 1.4894 (5.4) | 0.9907 (9.4) | 0.5081 (3.4) | 0.6194 (2.5) |
| Mode 3 | 0.400 | 0.5060 (5.0) | 0.4939 (4.3) | 0.4975 (4.9) | 0.5316 (5.2) | 0.4753 (4.8) | 0.3511 (5.2) | 0.4258 (4.9) |
| Time | -0.010 | -0.0119 (-3.6) | -0.0117 (-3.3) | -0.0108 (-3.3) | -0.0129 (-3.7) | -0.0100 (-3.2) | -0.0084 (-3.4) | -0.0098 (-3.2) |
| Cost | -0.004 | -0.0059 (-5.7) | -0.0058 (-5.4) | -0.0058 (-5.6) | -0.0062 (-5.8) | -0.0055 (-5.5) | -0.0045 (-5.3) | -0.0053 (-5.4) |
| High Income | 1.500 | 1.8401 (16.2) | 1.8395 (16.2) | 1.7462 (14.2) | 1.8419 (16.2) | 1.7351 (14.2) | 1.5089 (15.0) | 1.7539 (14.6) |
|  (SENL) |  |  | 1.0766 (2.6) |  |  |  |  |  |
| 22 (MNP) | 0.360 |  |  |  |  |  | 0.3433 (1.6) |  |
| 1 (HL) |  |  |  | 1.3309 (7.1) | 1.0 | 1.3646 (7.2) |  |  |
| 2 (HL) |  |  |  | 0.1248 (0.2) | 0.0845 (0.2) | 1.0 |  |  |
|  HEVL) |  |  |  |  |  |  |  | 0.6486 (3.9) |
| Average Log-Likelihood |  | -1.22208 | -1.22208 | -1.22049 | -1.22140 | -1.22098 | -1.21965 | -1.22109 |

It can be seen that the MNL model has some estimated parameters which are reasonably close to the reference values but larger deviations in the mode constants. The SENL results are not significantly different than those of the MNL; in fact, the structural parameter  is not significantly different from unity. On the other hand, the HL models, which correspond to a structure where the first two alternatives are grouped in a nest with structural parameter 1 and the rest in a nest with parameter 2. The results of these models are inconsistent as the first structural parameter is larger than one and the second is not significantly different from zero. This can be interpreted as empirical evidence against the notion that HL models can be used in the heteroscedastic case.

The MNP model has a slightly better log-likelihood and the parameters are well recovered (all reference values are within the confidence interval of the estimators), and the same situation occurs with the HEVL model. However the processing time necessary to estimate a HEVL is similar to that of the MNP, with an equally non optimised code.

The response analysis results indicate that only the last two (MNP and HEVL) models can reproduce accurately the response to policy changes. The changes applied were quite strong in some cases, such as multiplying or dividing the attributes by a factor of two. The MNL model yields large errors for strong policy changes, while the MNP and HEVL produced no significant errors, being slightly more accurate the MNP.

In the case of heteroscedasticity between observations (for example the RP/SP case), the value of 1=1.2 was applied to the first 1000 observations and the value 2=0.6 was used for the rest of them. The model estimation results are presented in Table 5. MNL1 and MNL2 refer to models estimated separately for each group of observations; the only difference between them being the scale parameter related to the variance. MNL2 has a better log-likelihood, probably due to the smaller variance of the error term. It is interesting to mention that when estimating a Logit model for the complete database (MNL) the estimated parameters are somewhere between those of MNL1 and MNL2. The SENL and MNP models yield accurate estimators, not significantly different from the reference values.

Table 5 : Models with heteroscedasticity between observations

|  | Ref. | MNL1 | MNL2 | MNL | SENL | MNP |
| --- | --- | --- | --- | --- | --- | --- |
| Mode 1 | -0.200 | -0.3001 (-1.3) | -0.4294 (-1.5) | -0.3110 (-1.8) | -0.2362 (-1.9) | -0.1418 (-1.4) |
| Mode 2 | 0.600 | 0.5913 (6.0) | 1.2535 (12.0) | 0.9121 (2.9) | 0.6269 (9.8) | 0.5798 (0.3) |
| Mode 3 | 0.400 | 0.3574 (2.7) | 0.8175 (5.4) | 0.5560 (5.6) | 0.4313 (5.7) | 0.3557 (6.0) |
| Time | -0.010 | -0.0101 (-2.1) | -0.0221 (-4.5) | -0.0156 (-4.6) | -0.0110 (-4.7) | -0.0105 (-5.1) |
| Cost | -0.004 | -0.0048 (-3.2) | -0.0106 (-6.1) | -0.0072 (-6.4) | -0.0053 (-6.4) | -0.0046 (-6.5) |
| High Income | 1.500 | 1.8552 (10.9) | 3.5053 (15.4) | 2.5545 (19.3) | 1.7910 (12.3) | 1.5252 (12.6) |
|  (SENL) |  |  |  |  | 1.9709 (11.0) |  |
| 22 (MNP) | 0.36 |  |  |  |  | 0.3690 (5.7) |
| Average Log-Likelihood |  | -1.28920 | -1.08936 | -1.20585 | -1.18945 | -1.18794 |
| Sample size |  | 1000 | 1000 | 2000 | 2000 | 2000 |

The response analysis gave surprising results: all models, including the MNL, can reproduce the response to policy changes accurately. All the prediction errors are under the threshold considered as the method error (variance of repetitions of the same experiment with different seeds). This means that although the theoretically correct models reproduce the situation accurately, so does (somehow) the MNL which ignores the presence of heteroscedasticity between observations. It is interesting to mention that this robustness of the MNL had been reported before for other types of departures from its basic assumptions (Williams and Ortúzar, 1982).

5.	CONCLUSIONS

We have shown that  multinomial Probit models are not so difficult to estimate neither are they always necessary. Its results are significantly better than those of more simple models in the presence of correlation, but only when there is a large number of observations available to estimate its parameters.

In the presence of heteroscedasticity between alternatives, the MNP and HEVL models behave well both seem to be equally difficult to estimate and the MNP is more flexible. As expected, the MNL, HL and SENL models fail to reproduce the response to strong policy changes in this case.

Surprisingly, in the presence of pure heteroscedasticity between observations the MNL model somehow manages to reproduce well the response to policy changes, even for the strongest policies tested. Obviously, the theoretically correct SENL and MNP models also behave well, and in these last models, the ratio between standard deviations is also accurately recovered.

ACKNOWLEDGEMENTS

We want to thank Ben Heydecker for his various contributions to this research project; although he did not participate directly in writing this paper we benefited greatly from discussions with him while preparing other joint publications. Finally, thanks are also due to the Chilean Fund for Scientific and Technological Development (FONDECYT) for having financed part of this research.

REFERENCES

Aptech Systems (1994) GAUSS User's Manuals. Maple Valley.

Ben-Akiva, M.E. and Lerman, S.R. (1985). Discrete Choice Analysis: Theory and Application to Travel Demand. The MIT press, Cambridge, Mass.

Ben-Akiva, M.E. and Morikawa, T. (1990) Estimation of travel demand models from multiple data sources. In M. Koshi (Ed.), Transportation and Traffic Theory. Elsevier, New York.

Bhat, C.R. (1995) A heteroscedastic extreme value model of intercity travel mode choice. Transportation Research 29B(6), 471-483.

Bhat, C.R. (1997) Covariance heterogeneity in nested logit models: econometric structure and application to intercity travel. Transportation Research 31B(1), 11-21.

Bolduc, D. (1994) A practical technique to estimate multinomial probit models in transportation: computational details and an application to a dissaggregate mode choice problem. Working Paper, Institut National de la Statistique et des Etudes Economiques, Quebec.

Börsch-Supan, A. (1990) Recent developments in flexible discrete choice models: nested logit analysis versus simulated moments probit analysis. In M.M. Fisher, P. Nijkamp and Y.Y. Papageorgiou  (Eds.), Behavioral Modelling of Spatial Choices and Processes. North-Holland, Amsterdam.

Börsch-Supan, A. and Hajivassiliou, V.A. (1993) Smooth unbiased multivariate probability simulators for maximum likelihood estimation of limited dependent variable models. Journal of Econometrics 58(3), 347-368.

Bradley, M.A. and Daly, A.J. (1991) Estimation of logit choice models using mixed stated preference and revealed preference information. Pre-prints Sixth International Conference on Travel Behaviour, 23-24 May 1991, Quebec, Canada.

Cirillo, C., Daly, A.J. and Lindveld, K. (1996) Eliminating bias due to repeated measurements problem in SP data. Proceedings 24th European Transport Forum, Brunel University, England, 2-6 September 1996.

Daganzo, C.F. (1979) Multinomial Probit: The Theory and its Applications to Demand Forecasting. Academic Press, New York.

Daly, A.J. (1987) Estimating "tree" logit models. Transportation Research 21B(4), 251-267.

Daly, A.J. (1992) ALOGIT 3.2. User's Guide. Hague Consulting Group, The Hague.

Greene, W.H. (1995) LIMDEP Version 7.0 User's Manual. Econometric Software Inc. Bellport, N.Y.

Gumbel, E.J. (1957) Statistics of Extremes. Columbia University Press, New York.

Lerman, S. and Manski, C. (1981) On the use of simulated frequencies to approximate choice probabilities. In C. Manski and D. McFadden (Eds.), Structural Analysis of Discrete Data With Econometric Applications. The MIT Press, Cambridge, Mass.

Mayberry, J.P. (1973) Structural requirements for abstract-mode models of passenger transportation. In R.E. Quandt (Ed.), The Demand for Travel: Theory and Measurement. D.C. Heath and Co., Lexington, Mass.

McFadden, D. (1973) Conditional logit analysis of qualitative choice behaviour. In P. Zarembka (Ed.), Frontiers in Econometrics, Academic Press, New York.

McFadden, D. (1989) A method of simulated moments for estimation of discrete response models without numerical integration. Econometrica 57(5), 995-1026.

Munizaga, M.A., Heydecker, B.G. and Ortúzar, J. de D. (1997) Representation of heteroscedasticity in discrete choice models. Transportation Research (under review).

Ortúzar, J. de D., Roncagliolo, D. and Velarde, U. (1997) Interactions and independence in stated preference modelling. Proceedings 25th European Transport Forum, Brunel University, England, 1-5 September 1997.

Ouwersloot, H. and Rietveld, P. (1996) Stated choice experiments with repeated observations. Journal of Transport Economics and Policy 30(2), 203-212.

Press, W.H., Teukolski, S.A., Vetterling, W.T. and Flannery, B.P. (1992) Numerical Recipes in Fortran 77: The Art of Scientific Computing. Second Edition, Cambridge University Press, Cambridge.

Williams, H.C.W.L. (1977) On the formation of travel demand models and economic evaluation measures of user benefit. Environment and Planning 9A(3), 285-344.

Williams H.C.W.L. and Ortúzar, J. de D. (1982) Behavioural theories of dispersion and the mis-specification of travel demand models. Transportation Research 16B(3), 167-219.

Yen, J.R., Mahmassani, H.S. and Herman, R. (1994) A model of employee participation in teleconmuting programs based on stated preference data. Preprints Seventh International Conference on Travel Behaviour, 799-813, Valle Nevado, Chile, 13-16 June 1994.

Notes

1	Heteroscedasticity is analysed in depth in a paper recently submitted to Transportation Research (Munizaga et al, 1997).